ECA1803 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO1 Engineering Design Assignment Report

Design of a Standards-Compliant Optical Fiber Telecommunication Link: From Architectural Specifications to Field Deployment

| Candidate Name: G.Asmitha | Register Number: 192512136 |
| --- | --- |
| Course Code & Title: ECA1803 – Optoelectronics and Optical Communication | Academic Program: B.Tech – Electronics and Communication Engineering |

1. Executive Summary & Design Requirement Statement

This technical report delineates a comprehensive, standards-compliant physical-layer engineering design for a point-to-point optical communication span linking a metropolitan Central Office (CO) to an enterprise corporate premises over an unrepeatered transmission distance of 60 km. The system is designed to operate within the 1550 nm telecommunication transmission window (C-band). It is specified to fulfill an initial transport capacity of 10 Gb/s utilizing intensity modulation and direct detection (IM/DD), while simultaneously establishing a robust, field-ready foundation capable of supporting future migration to 100 Gb/s coherent optical transmission without requiring cable infrastructure reinvestment.

The engineering methodology follows an analytical progression: validating electromagnetic waveguiding criteria, selecting standardized ITU-T fiber profiles, formulating end-to-end power margins, implementing chromatic dispersion compensation via Dispersion-Compensating Fiber (DCF), evaluating polarization-mode dispersion (PMD) constraints, and establishing an Amplified Spontaneous Emission (ASE) noise budget to guarantee targeted Optical Signal-to-Noise Ratio (OSNR) and Bit Error Rate (BER) benchmarks.

Core Design Specifications & Constraints:

Target Transmission Distance & Wavelength: 60 km end-to-end path length operating at nominal carrier wavelength λ = 1550 nm.

Data Rate & Upgrade Readiness: Baseline 10 Gb/s transmission (bit duration Tb = 100 ps), engineered to support 100 Gb/s (Tb = 10 ps) without physical route modification.

Fiber Selection & Bend Tolerance: Long-haul cable trunk using ITU-T G.652.D standard single-mode fiber, paired with ITU-T G.657.A bend-insensitive fiber at building ingress points to guarantee bend-induced excess loss ≤ 0.10 dB.

Pulse Integrity & Dispersion Compensation: Mitigation of accumulated group velocity dispersion (GVD) via an optimized DCF module to eliminate Inter-Symbol Interference (ISI) within the bit period.

Polarization Mode Dispersion Compliance: Total differential group delay (DGD) restricted to ≤ 0.10 Tb (PMD ≤ 10 ps at 10 Gb/s and ≤ 1.0 ps at 100 Gb/s).

Power Margin & Optical Signal-to-Noise Ratio: End-to-end link operating margin exceeding 6.0 dB relative to a -20.0 dBm receiver sensitivity, with a post-amplified OSNR ≥ 21.0 dB (exceeding the regulatory 17.0 dB threshold for BER ≤ 10^-12).

2. Stage 1: Interpretation of Communication Requirements & Governing Physics

Translating the 60 km, 10 Gb/s communication specification (with 100 Gb/s scalability) into an implementable hardware architecture necessitates an in-depth evaluation of the governing physical principles and optical propagation phenomena:

Selection of 1550 nm Operational Window: Silica-based optical fibers exhibit a global attenuation minimum in the third telecommunication window (~1550 nm), typically yielding an attenuation coefficient of ~0.18 to 0.20 dB/km. This spectral region coincides with the optical emission cross-section of Erbium-Doped Fiber Amplifiers (EDFAs), enabling wideband, low-noise amplification across the C-band without requiring complex electro-optic repeaters.

Electromagnetic Wave Optics & Optical Coherence: Because the optical core diameter (2a ≈ 8–9 μm) is of the same order of magnitude as the operational wavelength (1.55 μm), geometrical ray optics fails to capture diffraction, evanescent field decay, and waveguide modal behavior. Rigorous analysis requires Maxwell's electromagnetic wave equations, solved under weakly guiding approximations (Δ << 1) into Linearly Polarized (LP) mode solutions. High temporal and spatial coherence, provided by distributed-feedback (DFB) semiconductor laser diodes, is essential to sustain high data rates over 60 km.

Modal Confinement, Numerical Aperture, and V-Number: Light gathering and propagation modes are governed by the Numerical Aperture (NA) and the normalized frequency parameter (V-number). The cutoff parameter determines whether the waveguide remains strictly single-mode (preventing modal dispersion entirely) or allows higher-order transverse modes.

ITU-T Fiber Standards: Commercial deployment relies on standardized fiber categories. ITU-T G.652.D represents low-water-peak standard single-mode fiber (SSMF) with eliminated 1383 nm OH-ion absorption. ITU-T G.655 defines Non-Zero Dispersion-Shifted Fiber (NZ-DSF) optimized for suppressing nonlinear mixing. ITU-T G.657.A features trench-assisted refractive profiles to minimize macrobending radiation at tight entry radii.

Loss Mechanisms & Link Attenuation: Total optical attenuation comprises fundamental intrinsic losses (Rayleigh scattering scaling as 1/λ^4 contributing ~0.11 dB/km, and infrared lattice absorption contributing ~0.04 dB/km), extrinsic distributed losses (cabling and microbending at ~0.03 dB/km), and lumped localized losses (fusion splices, optical connectors, and building entrance bends).

Dispersion Mechanisms & Polarization Distortion: Chromatic dispersion (material and waveguide group velocity dispersion) induces temporal pulse spreading, causing pulse overlap and intersymbol interference (ISI) across adjacent bit slots. In addition, internal mechanical stress and slight core ellipticity induce modal birefringence, producing Polarization Mode Dispersion (PMD) that manifests as differential group delay (DGD).

OSNR and Signal Integrity: The receiver bit error rate (BER) is governed by the Optical Signal-to-Noise Ratio (OSNR) evaluated relative to Amplified Spontaneous Emission (ASE) noise generated by optical gain blocks. Maintaining an adequate OSNR margin above receiver detection thresholds is mandatory to ensure uncorrupted transmission.

3. Stage 2: Fiber Investigation & Propagation Configuration Selection

To determine the electromagnetic light-acceptance properties and modal structure of candidate dielectric waveguides, Candidate Fiber A and Candidate Fiber B are evaluated at λ = 1550 nm (1.55 μm).

The Numerical Aperture (NA) and the normalized frequency parameter (V-number) are defined mathematically by:

NA = sqrt(n1^2 - n2^2)

V = (2 * π * a / λ) * NA

where 'a' is the core radius, 'λ' is the vacuum wavelength, and n1 and n2 represent the core and cladding refractive indices, respectively.

| Candidate Fiber | Core Radius (a) | Refractive Indices | Numerical Aperture (NA) | V-Number (1550 nm) | Guided Modes (M) |
| --- | --- | --- | --- | --- | --- |
| Fiber A (Step-Index) | 4.0 μm | n1 = 1.4500 n2 = 1.4450 | 0.1203 | 1.9508 | 1 (Single-Mode, LP01 / HE11) |
| Fiber B (Step-Index) | 25.0 μm | n1 = 1.4800 n2 = 1.4650 | 0.2102 | 21.30 | ≈ 227 Modes (Multimode) |

Analytical Calculations & Modal Deductions:

1. Fiber A Analytical Derivation: 
   • Numerical Aperture: NA = sqrt(1.4500^2 - 1.4450^2) = sqrt(2.102500 - 2.088025) = sqrt(0.014475) = 0.120312 ≈ 0.1203.
   • Normalized Frequency: V = (2 * π * 4.0 μm / 1.55 μm) * 0.120312 = (25.1327 / 1.55) * 0.120312 = 16.2146 * 0.120312 = 1.9508.
   • Modal Cutoff Verification: For a cylindrical step-index optical waveguide, the cutoff for the second-order LP11 mode occurs at the first zero of the J0 Bessel function, V_c = 2.4048. Because V_A = 1.9508 < 2.405, higher-order transverse modes are completely cut off. Fiber A operates strictly in the fundamental single-mode regime (HE11 / LP01 mode). As a result, modal dispersion is non-existent.

2. Fiber B Analytical Derivation: 
   • Numerical Aperture: NA = sqrt(1.4800^2 - 1.4650^2) = sqrt(2.190400 - 2.146225) = sqrt(0.044175) = 0.210178 ≈ 0.2102.
   • Normalized Frequency: V = (2 * π * 25.0 μm / 1.55 μm) * 0.210178 = (157.0796 / 1.55) * 0.210178 = 101.3417 * 0.210178 = 21.2998 ≈ 21.30.
   • Guided Mode Count: For large-V multimode step-index fibers, the total number of bound modes is approximated by M ≈ V^2 / 2 = (21.2998)^2 / 2 = 453.68 / 2 ≈ 227 guided modes.
   • Modal Dispersion Impact: Supporting ~227 propagating modes leads to differential modal group delay on the order of several nanoseconds over a 60 km span. This level of intermodal delay completely destroys a 10 Gb/s signal (where bit period Tb = 100 ps), making Fiber B entirely unviable for high-speed transmission.

Wave Optics vs. Ray Optics Theoretical Justification:

The applicability of propagation theories is directly governed by the ratio of waveguide physical dimensions to the operating wavelength. In Fiber B, the core radius (a = 25.0 μm) is significantly greater than the carrier wavelength (a/λ ≈ 16.1). In this macroscopic regime, ray optics (geometric optics based on Fermat's principle and Snell's law of total internal reflection) offers a simple, intuitive, and accurate description of light propagation angles and numerical acceptance. Conversely, for Fiber A, the core radius (a = 4.0 μm) is directly comparable to the optical wavelength (a/λ ≈ 2.58). Geometrical ray optics fails in this microscopic domain because it neglects wave diffraction, phase shifts at dielectric boundaries, evanescent field penetration into the cladding, and polarization coupling. Consequently, electromagnetic wave optics (solving Maxwell's vector wave equations to obtain Bessel-derived LP mode eigenvalue solutions) is strictly mandatory to evaluate Fiber A.

Recommendation: Formal Design Decision 1 (Fiber Choice): Deploy ITU-T G.652.D standard single-mode fiber for the 60 km main transmission route, combined with ITU-T G.657.A bend-resilient fiber at the customer building entry. Candidate Fiber A satisfies the fundamental single-mode cutoff criterion (V = 1.9508 < 2.405), eliminating intermodal dispersion entirely while providing an exceptionally low attenuation profile (~0.18 dB/km) that guarantees 10 Gb/s integrity and provides an upgrade path to 100 Gb/s.

4. Stage 3: Development & Evaluation of the Optical Power Budget

To verify optical power feasibility, a backward power-budget calculation is conducted from receiver requirements. The optical link is bounded by a transmitter launch power of P_tx = +3.0 dBm (2.0 mW), a receiver sensitivity threshold of P_rx_sens = -20.0 dBm (0.01 mW), and a prescribed minimum operational safety margin of 6.0 dB. The total allowable channel loss is therefore P_tx - P_rx_sens = 23.0 dB, within which a ≥ 6.0 dB margin must be maintained.

| Link Parameter / Loss Mechanism | Standard G.652.D Link | Optimized G.657.A Entry + DCF Link |
| --- | --- | --- |
| Distributed Fiber Attenuation (0.18 dB/km × 60 km) | 10.80 dB | 10.80 dB |
| Optical Connectors (4 mated pairs × 0.40 dB) | 1.60 dB | 1.60 dB |
| Fusion Splices (10 splices × 0.08 dB per 6 km drum) | 0.80 dB | 0.80 dB |
| Premises / Building-Entry Macrobend Loss | 0.80 dB (Standard G.652.D) | 0.10 dB (Bend-Insensitive G.657.A) |
| Dispersion-Compensating Fiber (DCF) Module Loss | 0.00 dB (Uncompensated) | 5.10 dB (10.2 km @ 0.50 dB/km) |

Link Power Budget Evaluations:

1. Baseline Uncompensated Link (with G.657.A Building Entry):
   • Total Channel Attenuation: A_total = 10.80 dB + 1.60 dB + 0.80 dB + 0.10 dB = 13.30 dB.
   • Incident Optical Power at Receiver: P_rx = P_tx - A_total = +3.0 dBm - 13.30 dB = -10.30 dBm.
   • Effective System Margin: Margin = P_rx - P_rx_sens = -10.30 dBm - (-20.00 dBm) = 9.70 dB.
   • Compliance: The 9.70 dB margin comfortably exceeds the 6.0 dB design requirement by 3.70 dB, providing high protection against component aging and thermal drift.

2. Fully Dispersion-Compensated Link (incorporating 10.2 km DCF Module):
   • Total Channel Attenuation: A_total_DCF = 13.30 dB + 5.10 dB = 18.40 dB.
   • Incident Optical Power at Receiver: P_rx = +3.0 dBm - 18.40 dB = -15.40 dBm.
   • System Margin without Inline Gain: Margin = -15.40 dBm - (-20.00 dBm) = 4.60 dB.
   • Engineering Assessment: Under passive conditions, the 4.60 dB margin satisfies functional reception (P_rx > -20 dBm) but falls 1.40 dB short of the target 6.0 dB design margin. This reduction is resolved by incorporating an inline optical amplifier (+14.0 dB signal gain), which elevates the effective power level to -2.0 dBm and increases system headroom beyond 18 dB.

Dominant Loss Mechanism Analysis:

The 60 km fiber cable exhibits a total glass attenuation of 10.80 dB (0.18 dB/km), which breaks down into three distinct physical mechanisms: 
1. Rayleigh Scattering (0.11 dB/km): Accounts for 61.1% of intrinsic fiber loss (~6.60 dB total). This fundamental attenuation is caused by microscopic, sub-wavelength density fluctuations frozen into the amorphous silica glass matrix during preform cooling, scattering optical energy according to the 1/λ^4 relationship.
2. Intrinsic Infrared Absorption (0.04 dB/km): Accounts for 22.2% of glass loss (~2.40 dB total), representing the fundamental electronic bandgap tail and Si-O molecular bond vibrational resonances.
3. Distributed Microbending & Cabling Losses (0.03 dB/km): Contributes the remaining 16.7% (~1.80 dB total) caused by microscopic packaging deformities.
Lumped discrete losses (connectors, splices, and building bends) account for an additional 2.50 dB to 3.20 dB. By deploying ITU-T G.657.A fiber at the building ingress, macrobending loss is suppressed from 0.80 dB down to 0.10 dB, saving 0.70 dB.

Recommendation: Formal Design Decision 2 (Power Budget): Adopt the hybrid cabling topology utilizing ITU-T G.652.D for the trunk line and ITU-T G.657.A for enterprise entry routing. This reduces entry bend loss to 0.10 dB, yielding a passive received power of -10.30 dBm and a 9.70 dB system margin on the trunk. When integrated with the 10.2 km DCF module, the resulting 18.40 dB total loss is supplemented by optical amplification to maintain long-term reliability well beyond the 6.0 dB margin target.

5. Stage 4: Investigation & Selection of Dispersion / Polarization Management

High-speed digital transmission at 10 Gb/s and 100 Gb/s requires rigorous control over both Chromatic Dispersion (CD) and Polarization Mode Dispersion (PMD) to prevent temporal pulse spreading and severe intersymbol interference.

Chromatic Dispersion Broadening Analysis:

The temporal pulse broadening induced by group velocity dispersion is determined by:

Δτ_CD = D * Δλ * L

where D is the chromatic dispersion coefficient, Δλ is the source spectral linewidth (0.4 nm), and L is the transmission span (60 km). The total output pulse width is calculated using the Root-Sum-Square (RSS) formulation, assuming an initial transmitter pulse duration of τ_in = 30.0 ps:

τ_out = sqrt(τ_in^2 + Δτ_CD^2)

1. Standard ITU-T G.652.D Link: 
   • Dispersion Parameter: D = 17.0 ps/(nm·km) at 1550 nm.
   • Dispersion Delay Spread: Δτ_CD = 17.0 ps/(nm·km) * 0.4 nm * 60.0 km = 408.0 ps.
   • RSS Output Pulse Width: τ_out = sqrt((30.0)^2 + (408.0)^2) = sqrt(900 + 166464) = sqrt(167364) = 409.10 ps.
   • Impact on 10 Gb/s Transmission: At a 10 Gb/s line rate, the nominal bit slot duration is Tb = 1 / (10 × 10^9) = 100 ps. Because the output pulse width (409.10 ps) is more than four times larger than the bit period (409.10 ps >> 100 ps), the optical pulse spills across four adjacent bit intervals. This creates catastrophic Inter-Symbol Interference (ISI) and complete eye closure, making uncompensated transmission impossible.

2. Non-Zero Dispersion-Shifted Fiber (ITU-T G.655) Comparative Link: 
   • Dispersion Parameter: D = 4.5 ps/(nm·km) at 1550 nm.
   • Dispersion Delay Spread: Δτ_CD = 4.5 ps/(nm·km) * 0.4 nm * 60.0 km = 108.0 ps.
   • RSS Output Pulse Width: τ_out = sqrt((30.0)^2 + (108.0)^2) = sqrt(900 + 11664) = sqrt(12564) = 112.09 ps.
   • Assessment: Although G.655 reduces pulse broadening compared to G.652.D, the resulting 112.09 ps pulse width still exceeds the 100 ps bit slot, proving that dispersion management is mandatory regardless of fiber choice.

Dispersion-Compensating Fiber (DCF) Sizing & Synthesis:

To neutralize the accumulated chromatic dispersion of the G.652.D trunk (D_SMF * L_SMF = 17.0 ps/(nm·km) * 60 km = +1020 ps/nm), a module of Dispersion-Compensating Fiber with a negative dispersion parameter (D_DCF = -100 ps/(nm·km)) is inserted:

L_DCF = (D_SMF * L_SMF) / |D_DCF| = (17.0 * 60.0) / 100.0 = 1020 / 100.0 = 10.20 km

• DCF Insertion Loss: With an attenuation rate of 0.50 dB/km, the module introduces a localized insertion loss of A_DCF = 10.20 km * 0.50 dB/km = 5.10 dB.

Polarization-Mode Dispersion (PMD) & Modal Birefringence:

Because fiber fabrication introduces slight core non-circularities and asymmetric mechanical stresses, the fundamental LP01 mode splits into two orthogonal polarization states (HE11_x and HE11_y) that propagate with slightly different phase velocities. Over long distances, random polarization mode coupling leads to differential group delay (DGD) that scales with the square root of distance:

Δτ_PMD = D_PMD * sqrt(L)

For standard single-mode fiber with a PMD coefficient of D_PMD = 0.10 ps/√km across a 60 km span:

Δτ_PMD = 0.10 ps/√km * sqrt(60.0 km) = 0.10 * 7.74597 = 0.7746 ps ≈ 0.775 ps

System PMD Criterion Verification (Δτ_PMD ≤ 0.10 Tb):

10 Gb/s Operation: Bit slot Tb = 100 ps; allowable PMD limit = 0.10 * 100 ps = 10.0 ps. The link PMD of 0.775 ps is well below this threshold (0.775 ps << 10.0 ps), producing negligible power penalty (< 0.1 dB).

100 Gb/s Migration: Bit slot Tb = 10 ps; allowable single-carrier PMD limit = 0.10 * 10 ps = 1.0 ps. The link PMD of 0.775 ps remains strictly below the 1.0 ps ceiling. Furthermore, modern 100 Gb/s coherent systems utilize polarization-multiplexed QPSK (DP-QPSK) operating at baud rates of ~28–32 Gbaud with adaptive digital signal processor (DSP) equalization capable of tracking DGD well in excess of 10–20 ps. Thus, the physical glass infrastructure fully accommodates 100 Gb/s migration without hardware-based PMD compensators.

Optical Beat Length Evaluation: Modal birefringence is defined as B = |n_x - n_y|. For standard single-mode fiber with typical birefringence B = 1.0 × 10^-6, the optical beat length at λ = 1550 nm is:
L_B = λ / B = 1.55 × 10^-6 m / 1.0 × 10^-6 = 1.55 meters.
In polarization-maintaining (PM) fibers, strong artificial birefringence (B ≈ 10^-4) yields millimetric beat lengths (L_B ≈ 15 mm), locking polarization states against external perturbations.

Recommendation: Formal Design Decision 3 (Dispersion & PMD Strategy): Deploy ITU-T G.652.D standard fiber paired with a 10.2 km DCF module (D = -100 ps/(nm·km)). This cancels the 1020 ps/nm accumulated chromatic dispersion, restores pulse width well within the 100 ps bit duration, and preserves a total PMD of 0.775 ps—fulfilling both 10 Gb/s operation and the 100 Gb/s upgrade requirement.

6. Stage 5: Evaluation of OSNR & Optical Amplification Strategy

To support long-haul transmission and overcome component insertion losses, the Optical Signal-to-Noise Ratio (OSNR) is evaluated across an optical reference bandwidth of Δλ_ref = 0.10 nm (equivalent to ~12.5 GHz at 1550 nm). To sustain a Bit Error Rate (BER) ≤ 10^-12 in a 10 Gb/s direct-detection receiver, an OSNR threshold of ≥ 17.0 dB is required.

Pre-Amplification Optical Signal Conditions:

Prior to entering the inline optical amplifier, the unamplified signal levels are:
• Optical Signal Power: P_sig_in = -16.0 dBm
• Amplified Spontaneous Emission (ASE) Noise Power: P_ase_in = -39.0 dBm
• Input OSNR: OSNR_in = P_sig_in - P_ase_in = -16.0 dBm - (-39.0 dBm) = 23.0 dB.
The input OSNR (23.0 dB) exceeds the 17.0 dB threshold by 6.0 dB.

Optical Amplification Parameter Synthesis:

To compensate for the 18.40 dB total channel loss (including DCF) and maintain optimal optoelectronic receiver operation, an inline Erbium-Doped Fiber Amplifier (EDFA) is introduced with the following parameters:
• Signal Power Gain (G): +14.0 dB
• ASE Noise Growth: +16.0 dB (reflecting optical gain plus amplifier noise figure NF ≈ 5.0–5.5 dB).

Post-Amplification Link Calculations:

Applying the amplifier transfer parameters yields:
• Amplified Optical Signal Power: P_sig_out = P_sig_in + G = -16.0 dBm + 14.0 dB = -2.0 dBm.
• Amplified ASE Noise Power: P_ase_out = P_ase_in + ΔASE = -39.0 dBm + 16.0 dB = -23.0 dBm.
• Resulting Output OSNR: OSNR_out = P_sig_out - P_ase_out = -2.0 dBm - (-23.0 dBm) = 21.0 dB.
• Safety Margin Analysis: Margin_OSNR = OSNR_out - OSNR_req = 21.0 dB - 17.0 dB = 4.0 dB.

Engineering Quality Assessment: While the amplifier introduces a 2.0 dB OSNR penalty (reducing OSNR from 23.0 dB to 21.0 dB due to amplified spontaneous emission), the resulting output OSNR of 21.0 dB exceeds the 17.0 dB requirement with a 4.0 dB safety margin. This margin guarantees low-noise photo-detection, maintains Q-factors above 7.0 (BER < 10^-12), and ensures the boosted signal level (-2.0 dBm) remains within the linear response window of standard PIN/APD receivers without inducing photodetector saturation.

Recommendation: Formal Design Decision 4 (Amplification & OSNR): Approve the inline amplification scheme delivering 14 dB gain with a 16 dB ASE noise increase. The resulting 21.0 dB OSNR satisfies link performance criteria, ensures an operating safety margin of 4.0 dB above the 17.0 dB threshold, and provides the signal quality needed for error-free multi-gigabit transport.

7. Stage 6: Verification Matrix & Final Engineering Deployment Recommendation

To confirm overall system viability, each engineering parameter is validated against its specified operational threshold in the compliance matrix below:

| Engineering Requirement | Target Specification | Achieved Design Parameter | Compliance Status |
| --- | --- | --- | --- |
| Modal Condition & Fiber Classification | Single-mode operation (V < 2.405); ITU-T G.652.D / G.657.A | V = 1.9508 (< 2.405); G.652.D trunk + G.657.A drop | COMPLIANT (Pass) |
| Fiber & Link Attenuation Minimization | Minimize Rayleigh scattering, absorption, and macrobending | Cable: 10.80 dB (0.18 dB/km); Total link with DCF: 18.40 dB | COMPLIANT (Pass) |
| Received Optical Power & System Operating Margin | Sensitivity: -20.0 dBm; Operational Margin ≥ 6.0 dB | P_rx = -15.4 dBm (passive) / -2.0 dBm (amp); Margin: 4.6 dB (passive) / > 18 dB (boosted) | COMPLIANT (Pass) |
| Pulse Broadening & Dispersion Compensation | Maintain pulse integrity within bit slot (τ < 100 ps) | 10.2 km DCF neutralizes +1020 ps/nm; Pulse broadening eliminated | COMPLIANT (Pass) |
| PMD Performance & Upgrade Compatibility | PMD ≤ 0.10 Tb (≤ 10 ps at 10G; ≤ 1.0 ps at 100G) | Calculated PMD = 0.775 ps (< 1.0 ps single-carrier threshold) | COMPLIANT (Pass) |
| Optical Signal-to-Noise Ratio (OSNR) | Standard required OSNR ≥ 17.0 dB | Achieved OSNR = 21.0 dB (4.0 dB safety margin) | COMPLIANT (Pass) |

Final Engineering Deployment Decision:

Status: FINAL DEPLOYMENT DECISION: APPROVED FOR FIELD IMPLEMENTATION (YES)

Based on quantitative optical link modeling, electromagnetic wave theory, and ITU-T telecommunication standards, the proposed 60 km optical fiber link architecture satisfies all performance specifications. The deployment configuration resolves modal dispersion by enforcing single-mode confinement (V = 1.9508), mitigates premise installation bending loss using ITU-T G.657.A fiber (0.10 dB), neutralizes accumulated chromatic dispersion via a 10.2 km DCF module, constrains polarization mode dispersion to 0.775 ps (safely supporting 100 Gb/s transmission), and delivers an output OSNR of 21.0 dB (exceeding the required 17.0 dB threshold by 4.0 dB). The overall design establishes an operationally reliable 10 Gb/s transmission link with a clear migration path to 100 Gb/s coherent optical networking.